Please reference this figure for the question:

16.  Calculate the THROUGHPUT between Points B and G for the following cases:

a. The media is copper with a length of 150,000 meters.  The data rate is determined by multiplexing of 24-128kbps sources with a 10% overhead.  Frame length is 500 bits.  A Go-Back-N protocol is used (W=127) and the probability of error on the link is 10-8 ..  Assume BPSK modulation.

b. The media is free space using satellite communications.  The data is the same as in part a.  The distance is 70,000 km.  A Go-Back-N protocol is used (W=127) and the probability of error on the link is 10-5…. Assume BPSK modulation.